Simulating Superresolution Fluorescence Microscopy through Structured Illumination

Motivation

As technology continues to increase rapidly, the demands that such technology imposes continue likewise. As imaging technologies have increased over time, they have encountered the well-understood fundamental barrier of the diffraction limit. Nowhere is this more important than in the field of microscopy, where the never-ending urge to see ever-smaller presses against this limit daily. Fortunately, numerous methodologies have emerged to surpass this limit and should be explored and refined to continue these pursuits.

Simulation in particular offers the chance to examine these methodologies in a controlled environment, offering development of knowledge and understanding. Additionally, simulation, if properly designed, can act as a testing and proving ground for novel variations on the existing technology.

Related Work

The work of Gustafsson et al. has been of singular importance for the development of structured illumination microscopy (SIM) techniques. The work outlined in the 2000 paper, “Doubling the lateral resolution of wide-field fluorescence microscopy using structured illumination” will act as a roadmap for implementation of the initial simulation, as it details the mathematical theory behind the technique.

There has been a reasonable amount of work in the time since, offering another section of research from which to pull additional implementations.

Project Overview

We will first need to decide a stand-in ground truth image (or series of images) that represent the ideal, real-world image that could be acquired through an idealized unlimited imaging device. We will then simulate the image acquisition by a standard microscope objective by modelling the diffraction-limited resolution through convolution with the point spread function of a lens. Additional simulation should be pursued to model the dominant photon shot noise present in such low-light scenarios.

After creating an effective model of image capture through the modelled diffraction-limited optics, work can begin on the simulation of the structured illumination excitation. This simulation of the actual excitation can be managed by simple manipulation of the Fourier space of the images. As structured illumination at its simplest level relies on sinusoidal modulation, these operations can be well-modelled by mathematical operations in MATLAB.

From here we should have a simple model to run tests on. Tests might include deciding how many different illumination directions should be used to acquire some requisite quality, among others. Parameter optimization will be important for this, and in defining the apodization of the high frequency cutoff in the Fourier space of the combined image.

As the project proceeds, I would like to pursue implementation of more advanced algorithms, starting at using 2D structured illumination and continuing from there as time and sensibility allow.

Potential problems may arise due to inadequacies of the simulation process itself. Proper simulation relies on solid ground truth data, so the lack of ground truth can potentially present a simulated result not consistent with real-world results of the same application. So long as the constraints are noted, however, simulation can be a powerful tool toward proof of concept and rapid prototyping to test multiple algorithms at little to no cost.  Other problems may arise due to lack of knowledge in certain unforeseen areas, but I hope that my knowledge of 2D signal processing can be effective in navigating the difficulties.

Milestones, Timeline, and Goals

Week 1: Decide on test images. Research PSFs for microscopes. Design and finalize simulation of microscope capture. Begin work on implementation of basic SIM methodology with simple spatial sinusoids. Decide on comparison metrics. Meet with advisor (hopefully) to discuss any interesting horizons to explore in addition.

Week 2: Finalize any work on the basic SIM. Potentially implement a more advanced algorithm, starting with using 2D modulation, but possibly more.

Week 3: Make any final changes. Draw up poster design. Present.

References

Gustafsson, M. G. L., Agard, D. A. and Sedat, J. W. Doubling the lateral resolution of wide-field fluorescence microscopy using structured illumination. Proceedings of SPIE 3919: 141-150 (2000).

Gustafsson, M. G. L. Nonlinear structured-illumination microscopy: Wide-field fluorescence imaging with theoretically unlimited resolution. Proceedings of the National Academy of Sciences (USA) 102: 13081-13086 (2005).

Heintzmann, R. and Benedetti, P. A. High-resolution image reconstruction in fluorescence microscopy with patterned excitation. Applied Optics 45: 5037-5045 (2006).