A physics-based approach to modeling real-fuel combustion chemistry – VI. Predictive kinetic models of gasoline fuels

Rui Xu1, Chiara Saggese1, Robert Lawson2, Ashkan Movaghar2, Thomas Parise1, Jiankun Shao1, Rishav Choudhary1, Ji-Woong Park3, Tianfeng Lu3, Ronald K. Hanson1, David F. Davidson1,

Fokion N. Egolfopoulos2, Allen Aradi4, Arjun Prakash4, Vivek Raja Raj Mohan4, Roger Cracknell5,

Hai Wang1,*

1Department of Mechanical Engineering, Stanford University, Stanford, CA 94305, USA

2Department of Aerospace and Mechanical Engineering, University of Southern California, Los Angeles, CA 90089, USA

3Department of Mechanical Engineering, University of Connecticut, Storrs, CT 06269, USA

4Shell Global Solutions US Inc., 3333 Highway 6 South, Houston TX 77082, USA

5Shell Global Solutions (UK) 40 Bank Street, Canary Wharf, London E14 5NR, United Kingdom

*Corresponding Author Email: Allen.Aradi@shell.com; haiwang@stanford.edu

# S1. The evolution of the compositions of two gasoline fuels

Over the course of study, it was uncovered that the initial HyChem models developed from the fuel pyrolysis experiments were unable to reproduce the global combustion properties. The pyrolytic species measured were also found to be inconsistent with what would be expected from elemental compositions of the respective fuels. It was subsequently found that mishandlings of the fuels must have occurred during packaging and shipping: some of the low-molecular weight components have evaporated. As shown in Fig. S1, the actual compositions of the fuels (designated here as Shell A 2019 and Shell D 2019) differ from the original ones (designated here as Shell A 2017 and Shell D 2017) substantially. Some key properties of the fuels are also presented in Table S1. The original fuels were designated as E10 gasoline, containing slightly over 10% (wt) of ethanol. For the actual fuels tested, the ethanol contents had decreased significantly, to 6.7% (wt) and 5.5% (wt) in Shell A and Shell D, respectively. The reaction models proposed refer to fuels with the actual compositions tested, i.e., Shell A 2019 and Shell D 2019.

Fig. S1. Compositions (wt%) and basic properties of Shell A and D.

Table S1. Key properties of Shell A and D.

| Fuel | Composition | Average formula | wt% |  |  | MW (g/mol) | Mole fraction |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | C | H | O |  | HC | Ethanol |
| Shell A | 2019a | C6.97H14.36O0.15 | 83.3 | 14.4 | 2.3 | 100.5 | 0.855 | 0.145 |
|  | 2017 | C6.16H13.42O0.21 | 81.4 | 14.9 | 3.7 | 90.9 | 0.792 | 0.208 |
| Shell D | 2019a | C6.91H12.36O0.12 | 85.3 | 12.8 | 1.9 | 97.4 | 0.883 | 0.117 |
|  | 2017 | C5.93H11.57O0.19 | 82.9 | 13.6 | 3.5 | 85.9 | 0.811 | 0.189 |

a The composition of 2019 is used for the current work.

# S2. Additional physical and gas-phase thermochemical properties

We present a brief report of the thermochemical properties of the Shell gasoline A (2019) and D (2019), and their properties are listed in Table S1. The average molecular formula of each fuel is determined from the composition data (see, Table S3), including the mole fraction of ethanol and the mass fractions of C, H and O of the fuel as shown in Table S1. Also shown in the table are the mole fractions of ethanol and the hydrocarbon (HC) component of the fuel. As a supplementary information to Table S1, we present in Table S2 additional fuel physical and thermochemical properties, in Table S3 the fuel composition data in weight, volume, and molar percentages, and in Table S4 the distillation curve data of the two gasoline fuels.

For modeling purpose, we describe the fuel as an effective “two-component” mixture of ethanol and the HC component. Table S5 lists the average formula, molecular weight and heat of evaporation of the HC component of each fuel. The standard enthalpy of formation of the vapor-phase fuel may be determined from the lower heating value (LHV), enthalpy of evaporation (Hv) and molecular weight. Unfortunately, the latent enthalpy of evaporation is unavailable and has to be estimated from literature data. Following the same procedure described in the Supplementary material of Xu et al. [1], the Hv values of the HC component of each fuel are thus estimated and listed in Table S5. Then, the enthalpy of evaporation of the entire fuel (Hv,tot) can be estimated by the sum of Hv values of the HC component and ethanol, weighted by their respective mole fractions. Table S2 summarizes the lower heating values (LHV) and the vapor-phase standard enthalpy of formation of the two gasoline fuels. The Hv values are < 1% of the respective LHV values. Hence, the maximum error of 15% in an estimated Hv value leads to a negligible difference in the enthalpy of combustion of vapor-phase fuels (approximately 0.05 MJ/kg or < 0.15%).

Table S2. Additional physical and thermochemical properties for the Shell gasoline fuels.

| Fuel | Average formula | MW (g/mol) | Density @ 15°C (g/cm3) | LHV (MJ/kg) | Hv,tota (MJ/kg) | ΔfHo298b (kcal/mol) | Sulfur (wt %) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Shell A | C6.97H14.36O0.15 | 100.5 | 0.722 | 42.25 | 0.41 | –45.7 | < 0.001 |
| Shell D | C6.91H12.36O0.12 | 97.4 | 0.744 | 41.36 | 0.43 | –34.8 | < 0.001 |

a Estimated from the sum of enthalpy of evaporation of both ethanol and the HC component weighted by their respective mole fractions. b Derived from LHV, average formula and Hv.

Table S3. Composition data of Shell A and Shell D.

|  | Shell A |  |  | Shell D |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Group | Weight (%) | Volume (%) | Mole (%) | Weight (%) | Volume (%) | Mole (%) |
| n-paraffin | 5.4 | 5.5 | 4.7 | 9.5 | 10.2 | 7.6 |
| iso-paraffin | 65.1 | 68.1 | 60.3 | 29.6 | 33.3 | 29.1 |
| aromatics | 11.7 | 9.7 | 10.2 | 38.4 | 33.4 | 33.8 |
| naphthenes | 6.2 | 5.8 | 5.9 | 6.4 | 6.4 | 6.4 |
| olefins | 2.0 | 2.1 | 2.1 | 9.0 | 9.9 | 9.9 |
| oxygenate (ethanol) | 6.7 | 6.1 | 14.5 | 5.5 | 5.3 | 11.7 |
| unidentified | 2.9 | 2.7 | 2.3 | 1.6 | 1.5 | 1.5 |

Table S4. Distillation curve data of Shell A and D.

| Distillate vol. % off | Shell A Temperature (°C) | Shell D Temperature (°C) |
| --- | --- | --- |
| 0 | 26.1 | 26.2 |
| 5 | 27.3 | 36.3 |
| 10 | 63.5 | 60.3 |
| 15 | 77.2 | 63.4 |
| 20 | 88.7 | 70.3 |
| 25 | 98.5 | 78.2 |
| 30 | 98.7 | 87.8 |
| 35 | 98.9 | 98.3 |
| 40 | 99.1 | 105.1 |
| 45 | 106.2 | 110.4 |
| 50 | 109.7 | 110.5 |
| 55 | 112.4 | 113.5 |
| 60 | 113.2 | 124.1 |
| 65 | 114.3 | 138.3 |
| 70 | 117.5 | 144.2 |
| 75 | 125.0 | 160.7 |
| 80 | 143.1 | 167.9 |
| 85 | 161.7 | 176.1 |
| 90 | 176.1 | 187.5 |
| 95 | 184.0 | 200.4 |
| 100 | 207.3 | 235.4 |

Table S5. Key properties of the HC component.

| Fuel | HC component |  |  |
| --- | --- | --- | --- |
|  | Average formula | MW (g/mol) | Hva (MJ/kg) |
| Shell A | C7.82H15.79 | 109.8 | 0.37 |
| Shell D | C7.57H13.20 | 104.2 | 0.40 |

a Enthalpy of evaporation at 298K, estimated from equation:

Hv (kJ/mol) = 2.6 + 0.333 MW (g/mol)  + 10.9 yA, where yA is the total mass fraction of the aromatic compounds in the fuel mixture.

As we described in the main text, for modeling purpose, each fuel must be represented by a “binary” mixture of ethanol and the HC component given as CmHn where both m and n are integers.  The determination of m and n are based on the considerations of both the H/C ratio, the average molecular weight, and the average formula of the HC component given in Table S6. The formula of the modeled HC component is shown in Table S6 for each fuel, along with its enthalpy of formation. The thermochemical data of the HC component are developed here using the ΔfHo298 values listed and the specific heats and entropy to be discussed later.

Table S6. Description of the modeled fuels.

| Fuel | Modeled HC component |  |  |  |  |  | Mole fractionb |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Name | Formula | MW (g/mol) | ΔfHo298a (kcal/mol) | ΔfHo298/#C (kcal/mol) |  | HC | Ethanol |
| Shell A | SAC8H16 | C8H16 | 112.2 | –44.9 | –5.62 |  | 0.852 | 0.148 |
| Shell D | SDC8H14 | C8H14 | 110.2 | –33.8 | –4.23 |  | 0.877 | 0.123 |

a Derived from the ΔfHo298 of fuels and ethanol, as well as the HC and ethanol mole fractions in fuels. b These mole fraction values are slightly different from the actual values because of the use of approximate integer molecular formula for the HC component.

As we mentioned in the main manuscript, the use of integer molecular formula does introduce some difficulties in describing the fuel combustion chemistry, especially for gasoline fuels. In general, it is not possible to match both the equivalence ratio and mass fraction of the fuel at the same time because of small difference of the actual C/H/O ratio and the modeled C/H/O ratio. Here we take the middle ground. Specifically, using Shell A-air mixture as the example, the actual complete-combustion reaction is written as:

C6.97H14.36O0.15 + 10.49 O2 + 10.49 × (79/21) N2 = 6.97 CO2 + 7.18 H2O + 10.49 × (79/21) N2.

Decomposing the fuel into the HC component and ethanol, i.e., 0.855 C7.82H15.79 + 0.145 C2H5OH, the above formula is still exact.  Replacing C7.82H15.79 by the integer formula (C8H16), we have instead 0.837 C8H16 + 0.145 C2H5OH as the fuel mixture, where the value 0.837 comes from 0.855 × MW(C7.82H15.79)/ MW(C8H16). The approach taken here is equivalent to matching the mass fraction of the fuel between experiment and simulation. Re-normalization of the mole fractions of the “binary” fuel components yields 0.852 C8H16 + 0.148 C2H5OH, and these mole fraction values are given in the last two columns of Table S4.

The specific heat and entropy of each hydrocarbon may be estimated by defining a thermochemical surrogate, as shown in Table S7. For each hydrocarbon, a neat reference compound of molecular weight similar to the mean molecular weight of a class of similar compounds (e.g., iso-alkane) is assigned to that class. Attention has been placed on matching closely the average formula, mean molecular weight, and H/C ratio of the thermodynamic surrogate with those of the modeled hydrocarbon. Table S8 lists the thermochemical property values for the reference compounds considered (including ethanol). The thermochemical properties of the HC components of Shell A and D are listed in Tables S9 and S10, and the property data are presented in the form of NASA polynomials in Table S11.

Table S7. Thermochemical surrogate composition in mole fractions.

| Hydrocarbon Name | Composition | Average Formula |
| --- | --- | --- |
| SAC8H16 | 72.51% iso-octane (i-C8H18) + 7.09% methyl-cyclohexane (C7H14) + 12.32% toluene (C7H8) + 5.61% n-heptane (n-C7H16) + 2.48% 1-heptene (1-C7H14) | C7.73H16.27 |
| SDC8H14 | 33.50% iso-octane (i-C8H18) + 7.40% methyl-cyclohexane (C7H14) + 38.93% p-xylene (C8H10) + 8.77% n-heptane (n-C7H16) + 11.40% 1-heptene (1-C7H14) | C7.72H13.96 |

Table S8. Thermochemical data of reference species.

| Species | (cal/mol-K) | cp(T) (cal/mol-K) |  |  |  |  | Reference/ comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 300 K | 500 K | 1000 K | 1500 K | 2000 K |  |
| iso-octane | 101.1 | 45.3 | 70.0 | 108.7 | 129.1 | 142.0 | Therm [2] |
| methyl-cyclohexane | 82.4 | 30.6 | 53.5 | 90.1 | 102.1 | 109.2 | JetSurf 2.0 [3] |
| p-xylene | 80.7 | 28.5 | 47.4 | 74.2 | 85.5 | 92.0 | Burcat & Ruscic [4] |
| toluene | 76.5 | 24.8 | 40.6 | 62.5 | 71.6 | 76.8 | JetSurf 2.0 [3] |
| n-heptane | 102.2 | 39.9 | 60.1 | 93.1 | 106.1 | 114.0 | JetSurf 2.0 [3] |
| 1-heptene | 101.9 | 37.0 | 55.8 | 85.7 | 97.3 | 104.4 | JetSurf 2.0 [3] |
| ethanol | 67.1 | 15.7 | 22.8 | 34.2 | 39.5 | 42.7 | Burcat & Ruscic [4] |

Table S9. Thermochemical properties of SAC8H16 (Fuel A Hydrocarbon)

SAC8H16 = 8.0 C(S) + 8.0 H2 T Cp S H(T)-H(298) Hf Gf (K) (cal/mol-K) (cal/mol-k) (kcal/mol) (kcal/mol) (kcal/mol) 298. 40.953 96.841 0.000 -44.938 3.920 300. 41.207 97.115 0.082 -44.998 4.249 400. 53.392 110.665 4.821 -47.786 21.099 500. 64.318 123.779 10.718 -50.027 38.591 600. 73.784 136.365 17.636 -51.719 56.482 700. 81.797 148.358 25.427 -52.920 74.617 800. 88.569 159.734 33.954 -53.703 92.894 900. 94.520 170.515 43.113 -54.129 111.248 1000. 100.276 180.770 52.851 -54.201 129.631 1100. 104.479 190.528 63.092 -53.954 148.004 1200. 108.331 199.786 73.735 -53.484 166.345 1300. 111.856 208.598 84.747 -52.816 184.638 1400. 115.075 217.007 96.096 -51.973 202.873 1500. 118.011 225.049 107.753 -50.976 221.042 1600. 120.685 232.751 119.690 -49.844 239.141 1700. 123.114 240.142 131.882 -48.594 257.165 1800. 125.317 247.242 144.305 -47.242 275.112 1900. 127.313 254.072 156.938 -45.801 292.982 2000. 129.116 260.649 169.761 -44.286 310.773 2100. 130.741 266.989 182.755 -42.706 328.488 2200. 132.203 273.105 195.904 -41.073 346.125 2300. 133.515 279.011 209.191 -39.396 363.688 2400. 134.687 284.719 222.602 -37.683 381.177 2500. 135.732 290.239 236.124 -35.943 398.594

Table S10. Thermochemical properties of SDC8H14 (Fuel D Hydrocarbon)

SDC8H14 = 8.0 C(S) + 7.0 H2 T Cp S H(T)-H(298) Hf Gf (K) (cal/mol-K) (cal/mol-k) (kcal/mol) (kcal/mol) (kcal/mol) 298. 35.975 91.972 0.000 -33.838 7.164 300. 36.208 92.213 0.072 -33.895 7.440 400. 47.417 104.187 4.261 -36.537 21.628 500. 57.478 115.873 9.517 -38.719 36.431 600. 66.110 127.138 15.709 -40.437 51.630 700. 73.295 137.887 22.691 -41.747 67.084 800. 79.277 148.075 30.327 -42.717 82.701 900. 84.563 157.722 38.522 -43.394 98.421 1000. 89.923 166.904 47.242 -43.765 114.202 1100. 93.374 175.639 56.410 -43.866 130.005 1200. 96.538 183.901 65.908 -43.806 145.810 1300. 99.430 191.744 75.708 -43.603 161.604 1400. 102.067 199.211 85.785 -43.276 177.377 1500. 104.464 206.335 96.114 -42.840 193.124 1600. 106.636 213.148 106.670 -42.311 208.838 1700. 108.598 219.673 117.434 -41.704 224.516 1800. 110.364 225.931 128.383 -41.029 240.157 1900. 111.949 231.941 139.500 -40.300 255.759 2000. 113.366 237.720 150.767 -39.526 271.321 2100. 114.629 243.282 162.168 -38.717 286.844 2200. 115.750 248.641 173.688 -37.881 302.327 2300. 116.744 253.809 185.314 -37.026 317.772 2400. 117.621 258.796 197.033 -36.158 333.180 2500. 118.395 263.614 208.835 -35.285 348.551

Table S11. NASA Polynomials for hydrocarbons in fuels considered.

SAC8H16 S12/19C 8H 16 0 0G 298.000 3000.000 1 0.16599417E+02 0.48211332E-01-0.17137061E-04 0.30121534E-08-0.22484103E-12 2 -0.31723938E+05-0.64288055E+02 0.11043806E+01 0.60797900E-01 0.39996248E-04 3 -0.94742312E-07 0.43304766E-10-0.25833947E+05 0.23297089E+02 4

SDC8H14 S12/19C 8H 14 0 0G 298.000 3000.000 1 0.18020479E+02 0.37881661E-01-0.12259099E-04 0.16786289E-08-0.70531221E-13 2 -0.26540160E+05-0.72785660E+02 0.12218976E+01 0.46767473E-01 0.64798354E-04 3 -0.12255914E-06 0.55022525E-10-0.19829512E+05 0.23479656E+02 4

The adiabatic flame temperature (Tad) is calculated for the two gasoline fuels in air at 1 atm pressure. Fig. S2 show the adiabatic flame temperature plotted as a function of the equivalence ratio. Clearly, the Tad values of the two fuels are close to each other over the entire range of equivalence ratio, and peak within 2280-2305 K.  Given the small difference in the adiabatic flame temperature, it is unlikely that differences observed in the combustion behavior of these fuels, if any, is attributable to the difference in their thermochemical properties.

Fig. S2. Adiabatic flame temperature (Tad) computed for (a) Shell A and (b) Shell D in air (p = 1 atm, T0 = 298 K).

# References

[1] 	R. Xu, K. Wang, S. Banerjee, J. Shao, T. Parise, Y. Zhu, S. Wang, A. Movaghar, D.J. Lee, R. Zhao, X. Han, Y. Gao, T. Lu, K. Brezinsky, F. Egolfopoulos, D. Davidson, R. Hanson, C. Bowman, H. Wang, A physics-based approach to modeling real-fuel combustion chemistry – II. Reaction kinetic models of jet and rocket fuels, Combust. Flame 193 (2018) 520-537.

[2] 	E.R. Ritter, J.W. Bozzelli, THERM: Thermodynamic property estimation for gas phase radicals and molecules, Int. J. Chem. Kinet. 23 (1991) 767-778.

[3] 	H. Wang, E. Dames, B. Sirjean, D.A. Sheen, R. Tangko, A. Violi, J.Y.W. Lai, F.N. Egolfopoulos, D.F. Davidson, R.K. Hanson, C.T. Bowman, C.K. Law, W. Tsang, N.P. Cernansky, D.L. Millar, R.P. Lindstedt, A high-temperature chemical kinetic model of n-alkane (up to n-dodecane), cyclohexane, and methyl-, ethyl-, n-propyl and n-butyl-cyclohexane oxidation at high temperatures, JetSurF version 2.0, September 19, 2010 (http://web.stanford.edu/group/haiwanglab/JetSurF/JetSurF2.0/index.html), 2010.

[4] 	A. Burcat, B. Ruscic, Third millenium ideal gas and condensed phase thermochemical database for combustion (with update from active thermochemical tables), Argonne National Laboratory (ANL), 2005.